QQQ – Further Mechanics 1 – Chapter 3 – Elastic Strings, Springs & Hooke's Law (Version 1)

Total Marks: 23

(23 = Platinum, 21 = Gold, 18 = Silver, 16 = Bronze)

1.	 A light elastic string of natural length 0.4 m has one end A attached to a fixed point. The other end of the string is attached to a particle P of mass 2 kg. When P hangs in equilibrium vertically below A, the length of the string is 0.56 m.

(a)	 Find the modulus of elasticity of the string. 	(3)

A horizontal force is applied to P so that it is held in equilibrium with the string making an angle θ with the downward vertical. The length of the string is now 0.72 m.

(b)	 Find the angle θ. 	(3)

(Total 6 marks)

Edexcel M3 January 2008 Q1

2.

Figure 1

A particle of mass 0.8 kg is attached to one end of a light elastic spring, of natural length 2 m and modulus of elasticity 20 N. The other end of the spring is attached to a fixed point O on a smooth plane which is inclined at an angle α to the horizontal, where tan⁡α=34. The particle is held on the plane at a point which is 1.6 m down a line of greatest slope of the plane from O, as shown in Figure 1. The particle is then released from rest.

Find the initial acceleration of the particle. 	(6)

(Total 6 marks)

Edexcel M3 June 2005 Q1

3.	 A light elastic string with natural length l and modulus of elasticity kmg has one end attached to a fixed point A on a rough inclined plane. The other end of the string is attached to a package of mass m.

The plane is inclined at an angle θ to the horizontal, where tan⁡θ=512

The package is initially held at A. The package is then projected with speed 6gl up a line of greatest slope of the plane and first comes to rest at the point B, where AB=3l.

The coefficient of friction between the package and the plane is 14

By modelling the package as a particle,

(a)	 show that k=1526 	(6)

(b)	 find the acceleration of the package at the instant it starts to move back down the plane from the point B. 	(5)

(Total 11 marks)

Edexcel A2 October 2020 Q6

Solutions

1.

Mark scheme (a)

| Scheme | Marks |
| --- | --- |
| T or λ×el=mg (even T=m is M1, A0, A0 sp case) | M1 |
| λ×0.160.4=2g | A1 |
| ⇒λ=49N or 5g | A1 |
|  | (3) |

Mark scheme (b)

| Scheme | Marks |
| --- | --- |
| R(↑) Tcos⁡θ=mg or cos⁡θ=mgT | M1 |
| 49.0.320.4.cos⁡θ=19.6 or 4g.cos⁡θ=2g or 2mg.cos⁡θ=mg (ft on their λ) | A1ft |
| ⇒cos⁡θ=12⇒θ=60∘ (or π3 radians) | A1 |
|  | (3) |
|  | (6 marks) |

Notes

Special case Tsin⁡θ=mg giving θ=30 is M1 A0 A0 unless there is evidence that they think θ is with horizontal – then M1 A1 A0

2.

| Scheme | Marks |
| --- | --- |
| HL T=20×0.42 (=4) accept −4 | M1 A1 |
| mgsin⁡α+T=ma | M1 A1 |
| 0.8g×0.6+4=0.8a | M1 |
| a=10.88≈10.9 (m s−2) accept 11 | A1 |
|  | (6 marks) |

Notes

The scheme brackets the last two M1 marks: the M1 for 0.8g×0.6+4=0.8a depends on the previous M1.

3.

Mark scheme (a)

| Scheme | Marks | AO |
| --- | --- | --- |
| Work done against friction =3l×μmgcos⁡θ (=9mgl13) Gain in EPE =kmg×4l22l (=2kmgl) Gain in GPE =mg×3lsin⁡θ (=15mgl13) | B1 B1 B1 | 3.4 3.4 3.4 |
| Work energy equation: | M1 | 2.1 |
| 12m×6gl=9mgl13+2kmgl+15mgl13 | A1 | 1.1b |
| 2k=3−2413=1513, k=1526 * | A1* | 2.2a |
|  | (6) |  |

Notes

B1 B1 B1: Use model to obtain one correct term
Use model to obtain two correct terms
Use model to obtain three correct terms

M1: Work-energy equation. Need all terms and no extras. Dimensionally correct. Condone sign errors and sin/cos confusion.

A1: Correct unsimplified equation

A1*: Obtain given result from correct working

NB: The use of suvat equations is not a valid alternative method because the acceleration is not constant

Mark scheme (b)

| Scheme | Marks | AO |
| --- | --- | --- |
| Tension in the string at B: 1526mg×2ll (=15mg13) | B1 | 3.1a |
| Equation of motion: tension + component of weight − friction =ma | M1 | 3.3 |
| 15mg13+mgsin⁡θ−14mgcos⁡θ=ma (mg(1513+513−313)=ma) | A1 A1 | 1.1b 1.1b |
| a=17g13 | A1 | 1.1b |
|  | (5) |  |
|  | (11 marks) |  |

Notes

B1: Correct unsimplified expression for the tension in the string

M1: Equation of motion. Need all terms and no extras. Condone sign errors and sin/cos confusion Allow with T or their T

A1 A1: Unsimplified equation with at most one error
Correct unsimplified equation

A1: Exact answer or accept 12.8 or 13 (m s−2)